APPENDIX (Software)

ARDUINO CODE WITHOUT TEMPERATURE SENSING

/*

State change detection (edge detection)

This program monitors a rodent tail flick and records the

time to a file specified by the user

The circuit:

* phototransistor that sets pin 2 high when activated

* connects 5V to an electronic relay when activated

* the electronic relay controls the heat lamp

created  27 Sep 2010

modified 26 Oct 2010

by Aaron Otto

This example code is based on

http://arduino.cc/en/Tutorial/ButtonStateChange

*/

//include libraries for reading and writing to the serial port

#include <SoftwareSerial.h>

// this constant won't change:

const int  buttonPin = 2;    // the pin that the pushbutton is attached to

const int ledPin = 12;       // the pin that the LED is attached to

int val = 0;                 // variable for reading the pin status

char msg = ' ';              //variable to hold data from serial

// Variables will change:

int buttonPushCounter = 0;   // counter for the number of button presses

int buttonState = 0;         // current state of the button

int lastButtonState = 0;     // previous state of the button

unsigned long time, LoopStart;

extern volatile unsigned long timer0_overflow_count;

unsigned long waitTime = LoopStart + 30000;

void setup() {

// initialize the button pin as a input:

pinMode(buttonPin, INPUT);

// initialize the LED as an output:

pinMode(ledPin, OUTPUT);

// initialize serial communication:

Serial.begin(9600);

Serial.print("Program Initialized\n");

//keeps light off

digitalWrite(ledPin, HIGH);

}

void loop() {

// read the pushbutton input pin:

buttonState = digitalRead(buttonPin);

while (Serial.available()>0){

msg=Serial.read();

}

// compare the buttonState to its previous state

if (msg=='Y') {

//turns light on

digitalWrite(ledPin, LOW);

if (buttonState != lastButtonState) {

if (buttonState == HIGH) { //no light is seen

digitalWrite(ledPin, LOW); //keep the relay closed for heat lamp on

LoopStart = millis();

msg = 'Y';

}

else { //the sensor sees light turning pin 2 LOW

digitalWrite(ledPin, HIGH);

time = millis() - LoopStart;

//prints time since program started

msg='N';

Serial.println(time/1000.);

}

}

// save the current state as the last state,

//for next time through the loop

lastButtonState = buttonState;

}

else {

}      // if message is anything other than Y do nothing

}

ARDUINO CODE WITH TWO TEMPERATURE SENSING

/*

State change detection (edge detection)

This program monitors a rodent tail flick and records the

time to a file specified by the user

The circuit:

* phototransistor that sets pin 2 high when activated

* connects 5V to an electronic relay when activated

* the electronic relay controls the heat lamp

created  27 Sep 2010

modified 26 Oct 2010

by Aaron Otto

This example code is based on

http://arduino.cc/en/Tutorial/ButtonStateChange

*/

//include libraries for reading and writing to the serial port

#include <math.h>

#include <SoftwareSerial.h>

// this constant won't change:

const int  sensePin = 2;    // the pin that the phototransistor is attached to

const int relayPin = 12;       // the pin that the relay is attached to

int val = 0;                 // variable for reading the pin status

char msg = ' ';              //variable to hold data from serial

// Variables will change:

int senseState = 0;         // current state of the phototransistor

int lastsenseState = 0;     // previous state of the phototransistor

unsigned long time, LoopStart;

extern volatile unsigned long timer0_overflow_count;

unsigned long waitTime = LoopStart + 30000;

/* Thermistor

* Inputs ADC Value from Thermistor and outputs Temperature in Celsius

*  requires: include <math.h>

* Utilizes the Steinhart-Hart Thermistor Equation:

* Schematic:

*   [Ground] -- [10k-pad-resistor] -- | -- [thermistor] --[Vcc (5 or 3.3v)]

*                                     |

*                                Analog Pin 1/2

*

* In case it isn't obvious (as it wasn't to me until I thought about it), the analog ports

* measure a 10-bit digitized voltage between 0v -> Vcc which for an Arduino is a nominal 5V

*

* The resistance calculation uses the ratio of the Vcc to the Vpad. This ensures that if there are

* variations in Vcc they are taken care of in the calculation.

* Resistance = (Vcc(ADC0)/Vpad(ADC1 or 2) - 1) * Rpad

*/

double Thermistor(int RawADC1, int RawADC2) {

double Temp, Resistance, Volts, nom_R=9780., dummy;

Volts = double(RawADC2) / 1024. * 5.; //check the Arduino voltage for adjusting (not yet implemented)

dummy = double(RawADC2)/double(RawADC1); //Vcc/Vpad

// Resistance = log(10240000/RawADC1 - 10000);

Resistance = log((dummy - 1.) * nom_R); //This is the actual resistance value since it uses the total Arduino voltage

//Datasheet data fitted in Igor as T(C) vs. R(ohms)

Temp = 362.62 - 69.556 * Resistance + 4.3026 * Resistance*Resistance - 0.11392 * Resistance*Resistance*Resistance;

//Temp = Temp - 273.15;            // Convert Kelvin to Celcius

// Temp = (Temp * 9.0)/ 5.0 + 32.0; // Convert Celcius to Fahrenheit

return Temp;

}

void setup() {

// initialize the sense pin as a input:

pinMode(sensePin, INPUT);

// initialize the relay pin as an output:

pinMode(relayPin, OUTPUT);

// initialize serial communication:

Serial.begin(9600);

Serial.print("Program Initialized\n");

//keeps light off

digitalWrite(relayPin, HIGH);

}

void loop() {

// read the sense (phototransistor) input pin:

senseState = digitalRead(sensePin);

while (Serial.available()>0){

msg=Serial.read();

}

// compare the senseState to its previous state

if (msg=='Y') {

//turns light on

digitalWrite(relayPin, LOW);

if(millis() % 1000 > 950){

Serial.print(Thermistor(analogRead(1), analogRead(0)), 1);

Serial.print(", ");

Serial.println(Thermistor(analogRead(2), analogRead(0)), 1);

}

if (senseState != lastsenseState) {

if (senseState == HIGH) { //no light is seen

digitalWrite(relayPin, LOW); //keep the relay closed for heat lamp on

LoopStart = millis();

msg = 'Y';

}

else { //the sensor sees light causing pin 2 to be LOW

digitalWrite(relayPin, HIGH);//switch the relay to high, turning the light off

time = millis() - LoopStart;

//prints time since program started

msg='N';

//Serial.println(time/1000.);

//Serial.flush();

Serial.print("flick");

}

}

// save the current state as the last state,

//for next time through the loop

lastsenseState = senseState;

}

else {

}      // if message is anything other than Y do nothing

}

PYTHON (GUI) CODE WITHOUT TEMPERATURE SENSING

from Tkinter import *

import time

import serial

dt = 0

locations=[2, 3, 4, '/dev/ttyUSB0','/dev/ttyUSB1','/dev/ttyUSB2','/dev/ttyUSB3',

'/dev/ttyS0','/dev/ttyS1','/dev/ttyS2','/dev/ttyS3']

for device in locations:

try:

print "Trying...",device

ser = serial.Serial(device, 9600, timeout=1)

time.sleep(1)

device_feedback = ser.readline()

if device_feedback == "Program Initialized":

print device_feedback

elif device_feedback == "":

print "Port ", device, " active but not Arduino."

continue

else:

break

break

except:

print "Failed to connect on",device

ser.flushOutput()

ser.flushInput()

class MyDialog:

def __init__(self,parent):

global dt, samplename

self.entryVariable = StringVar()

self.entryVariable.set(str(dt))

self.sample = StringVar()

self.sample.set("Mouse 1")

self.filename = StringVar()

self.filename.set("temp.csv")

global top

top = self.top = parent

self.e = Entry(top, textvariable = self.filename)

self.e.config(font=("Helvetica", 12),state=NORMAL)

self.e2 = Entry(top, textvariable = self.sample)

self.e2.config(font=("Helvetica", 12),state=NORMAL)

self.e3 = Entry(top, textvariable = self.entryVariable)

self.e3.config(font=("Helvetica", 12), justify=CENTER,state=NORMAL)

Label(top, text="What is the file name?", font=("Helvetica",12)).pack()

self.e.pack(padx=50, pady=5, fill=X)

Label(top, text="What is the sample?", font=("Helvetica",12)).pack()

self.e2.pack(padx=3, pady=5)

Label(top, text="elapsed time (s)", font=("Helvetica",12)).pack()

self.e3.pack(padx=3, pady=5)

self.b = Button(top, text="OK", command=self.ok, font=("Helvetica",12))

self.b.pack(pady=5, side=BOTTOM)

self.e.bind("<Return>", self.ok)

self.e.bind("<Escape>", self.escape)

def ok(self, event = None):

samplename = self.e2.get()

ser.write('Y')

start = time.time()

filename = self.e.get()

if len(filename) == 0:

filename = self.filname

self.e = Entry(top, textvariable = self.filename)

root.update()

print "Your file is ", filename

f=open(filename, 'a')

line = ""

ser.flushOutput()

ser.flushInput()

while(line == ""):

line = ser.readline()       #why does readline and not read get data

dt = time.time()-start

self.entryVariable.set("%.0f" % dt)

self.e3 = Entry(top, textvariable = self.entryVariable)

root.update()

dt = time.time()-start

ser.flushOutput()

ser.flushInput()

f.writelines("%s, %.1f,\r" % (samplename, dt))

f.close()

print "%.1f" % dt

self.entryVariable.set("%.1f" % dt)

return dt

def escape(self, event = None):

print "Exiting the program"

ser.close()

root.destroy()

return 0

root = Tk()

root.title("Tail Flick Analgesic Meter")

d = MyDialog(root)

root.wait_window(d.top)

PYTHON (GUI) CODE WITH TWO TEMPERATURE SENSING

from Tkinter import *

import time

import serial

dt = 0

locations=[2, 3, 4, '/dev/ttyUSB0','/dev/ttyUSB1','/dev/ttyUSB2','/dev/ttyUSB3',

'/dev/ttyS0','/dev/ttyS1','/dev/ttyS2','/dev/ttyS3']

for device in locations:

try:

print "Trying...",device

ser = serial.Serial(device, 9600, timeout=1)

time.sleep(1)

device_feedback = ser.readline()

if device_feedback == "Program Initialized":

print device_feedback

elif device_feedback == "":

print "Port ", device, " active but not Arduino."

continue

else:

break

break

except:

print "Failed to connect on",device

ser.flushOutput()

ser.flushInput()

class MyDialog:

def __init__(self,parent):

global dt, samplename

self.entryVariable = StringVar()

self.entryVariable.set(str(dt))

self.T1Variable = StringVar()

self.T2Variable = StringVar()

self.sample = StringVar()

self.sample.set("Mouse 1")

self.filename = StringVar()

self.filename.set("temp.csv")

global top

top = self.top = parent

self.e = Entry(top, textvariable = self.filename)

self.e.config(font=("Helvetica", 12),state=NORMAL)

self.e2 = Entry(top, textvariable = self.sample)

self.e2.config(font=("Helvetica", 12),state=NORMAL)

self.e3 = Entry(top, textvariable = self.entryVariable)

self.e3.config(font=("Helvetica", 12), justify=CENTER,state=NORMAL)

self.e4 = Entry(top, textvariable = self.T1Variable)

self.e4.config(font=("Helvetica", 12), justify=CENTER,state=NORMAL)

Label(top, text="What is the file name?", font=("Helvetica",12)).pack()

self.e.pack(padx=50, pady=5, fill=X)

Label(top, text="What is the sample?", font=("Helvetica",12)).pack()

self.e2.pack(padx=3, pady=5)

Label(top, text="elapsed time (s)", font=("Helvetica",12)).pack()

self.e3.pack(padx=3, pady=5)

Label(top, text="Temperature (C)", font=("Helvetica",12)).pack()

self.e4.pack(padx=3, pady=5)

self.b = Button(top, text="OK", command=self.ok, font=("Helvetica",12))

self.b.pack(pady=5, side=BOTTOM)

self.e.bind("<Return>", self.ok)

self.e.bind("<Escape>", self.escape)

def ok(self, event = None):

samplename = self.e2.get()

ser.write('Y')

start = time.time()

filename = self.e.get()

if len(filename) == 0:

filename = self.filname

self.e = Entry(top, textvariable = self.filename)

root.update()

print "Your file is ", filename

f=open(filename, 'a')

#line = ""

ser.flushOutput()

ser.flushInput()

temperature = []

times = []

line = []

while(line != "flick"):

line = ser.readline()       #why does readline and not read get data

#print line[0]

dt = time.time()-start

times.append(dt)

temperature.append(line)

self.entryVariable.set("%.0f" % dt)

self.e3 = Entry(top, textvariable = self.entryVariable)

self.T1Variable.set("%s" % line)

self.e4 = Entry(top, textvariable = self.T1Variable)

root.update()

dt = time.time()-start

ser.flushOutput()

ser.flushInput()

f.writelines("\r%s, %.1f\r" % (samplename, dt))

index = 0

for x in times:

f.writelines("%s, " % x)

f.writelines(temperature[index])

index += 1

f.close()

#        print "%.1f" % dt

self.entryVariable.set("%.1f" % dt)

return dt

def escape(self, event = None):

print "Exiting the program"

ser.close()

root.destroy()

return 0

root = Tk()

root.title("Tail Flick Analgesic Meter")

d = MyDialog(root)

root.wait_window(d.top)